Sorting & processing the catch

Crew members must be instructed about the potential risks when sorting and processing the catch. Consider, for example, the use of knives and gloves, and pay attention to poisonous/dangerous marine life that can cause painful injuries. Stones, ammunition and other objects in the catch can also cause injuries.

Threats

- Cuts and injuries

- Physical overload

- Slipping and getting knocked overboard

- Poisoning

Step 1: Collective measures

Collective measures

- Provide a conveyor belt at the correct height to limit physical strain

- Instruct the crew how to safely handle knifes

- Instruct the crew about the symptoms of a weever sting (see appendix sorting & processing the catch)

- Instruct the crew how to handle objects such as stones and ammunition on the conveyor belt

- Regularly rotate the work

- Ensure a stable and non-slip surface

- The emergency button of the conveyor belt must be visible and within reach from the workplace

Step 3: Personal protective equipment

- Use safety gloves and safety footwear

- Do not wear jewellery, prevent your hair from getting stuck, wear fitting work clothes

- Use lifejackets when working on deck

Step 2: Individual measures

- Always check knifes before use

- Store knives safely

- Adopt a stable working posture (bent knees, two feet on the ground)

- Keep a close eye on the conveyor belt for foreign objects and warn colleagues in time verbally